Free Ride

Directions ~ Givens ~ Inquiry Questions~ Generalizations

Directions/Controls:

- Select a rear gear; Select a front gear

- Click pedal for a half pedal turn – forward or backward

- Choose Free Ride or a Route (five flags you try to collect)

- Reset button

Given:

- Q: What does the number on the gear mean?  A: Circumference.

As a Game, What is the Goal?

- Try to collect all five flags with the fewest number of half pedal turns.

Inquiry Questions

- As the front gear gets bigger how does this affect the distance?  Goes farther

- As the rear gear gets bigger how does this affect the distance?  Less distance

- What is the largest distance you can go with one-half pedal turn?  two

- What is the shortest distance you can go with one-half pedal turn?  One-third

- Front gear same as rear means half turn means a distance of one-half

- Front gear larger than rear means half turn goes farther than one-half in distance

- Front gear smaller than rear means have turn goes less than one-half in distance

- What is the gear ratio?  Front circumference over rear circumference

- **Given the gear ratio, what is the distance for half a pedal? One-half times the gear ratio is the distance traveled

- ** What are all the distances we can go with one-half pedal turn?

  - What are good ways to list all the possible distances?  (A) Make a table with the front gear across the top and rear gear down the side-put the distances in the body of the table (shown in the generalizations below).  (B) Make a three-column table with distances in the first column listed from smallest to largest, ratios in second column, gear numbers in the third column.

Extensions

- Energy input?  Power…work.

Generalizations

- Given the gear ratio, what is the distance for half a pedal? One-half times the gear ratio is the distance traveled by a one-half pedal turn

- All the possible distances with one-half pedal turn:

|  |  | Front Gear |  |  |
| --- | --- | --- | --- | --- |
|  |  | 20 | 30 | 40 |
| Rear Gear | 10 | 1 | 1.5 | 2 |
|  | 15 | 2/3 | 1 | 1 1/3 |
|  | 20 | 1/2 | 3/4 | 1 |
|  | 25 | 2/5 | 3/5 | 4/5 |
|  | 30 | 1/3 | 1/2 | 2/3 |

| Distance | Ratio | Front:Rear |
| --- | --- | --- |
| 1/3 | 2:3 | 20:30 |
| 2/5 | 4:5 | 20:25 |
| 1/2 | 1:1 | 30:30 “x:x” |
| 3/5 | 6:5 | 30:25 |
| 2/3 | 4:3 | 40:30 |
| 3/4 | 3:2 | 30:20 |
| 4/5 | 8:5 | 40:25 |
| 1 | 2:1 | 20:10 “2x:x” |
| 1 1/3 | 8:3 | 40:15 |
| 1 1/2 | 3:1 | 30:10 |
| 2 | 4:1 | 40:10 |